# Jetson Orin Nano Setup Guide (JetPack Install ~ Boot Logo Customization)

Target hardware: Jetson Orin Nano Super Developer Kit (P3767-0005 module / P3768-0000 carrier board) Installed version: JetPack 7.2 / L4T R39.2.0 (Ubuntu 24.04 based) Host PC: Ubuntu (x86_64) End result: Replace every NVIDIA logo with the company logo (LS Automotive) from power-on through desktop, plus auto-login and auto-launching a GUI app.

Fig. Final result — company logo at all 4 stages (UEFI / Plymouth / Login / Desktop)

## 0. Big Picture (4 stages where the logo appears)

On boot, the NVIDIA logo appears in four different places, each from a different source, so each needs a different fix.

| Stage | When it shows | Source | Fix |
| --- | --- | --- | --- |
| 1. UEFI firmware | Right after power-on (ESC to enter Setup screen) | BMP inside QSPI firmware | Build UEFI from source, flash QSPI |
| 2. Plymouth | During kernel boot | Plymouth theme in rootfs | Replace theme + regenerate initrd |
| 3. Post-login (~8s) | Right after login, before desktop | ~/.xsessionrc paints it via feh | Replace background PNG |
| 4. Desktop background | On the desktop | gsettings wallpaper | Change gsettings |

## 1. Install JetPack 7.2 (SDK Manager)

### 1-1. Install SDK Manager (host PC)

- developer.nvidia.com → search “NVIDIA SDK Manager” → log in (NVIDIA developer account) → download .deb

- Install:

- sudo apt install ~/Downloads/sdkmanager_*.deb

  - sudo: admin privileges (required to install), will prompt for password

  - apt install: Ubuntu’s package install command

### 1-2. Put the Jetson into Force Recovery mode

To flash, the board must be in “recovery mode”. 1. Fully power off the Jetson (disconnect power cable) 2. Connect Jetson ↔ host PC with a USB-C cable 3. Hold the REC button while reconnecting power (or pressing the power button) 4. Release REC after 2~3 seconds

Fig. Jetson board (underside — microSD slot / buttons / connectors)

Verify (host PC terminal):

lsusb | grep -i 0955

- 0955:7523 NVIDIA Corp. APX → recovery mode success ✅

- 0955:7020 ... L4T running on Tegra → it just booted normally (failed); retry REC timing

### 1-3. ⚠️ SDK Manager GUI crash → work around with CLI (important pitfall)

On recent host OSes the sdkmanager GUI may crash immediately (SIGSEGV). When reinstalling/sandbox options don’t help, use CLI mode (this is the key trick):

sdkmanager --query interactive --product Jetson

- ⚠️ Never attach a pipe like | head — the pipe removes the TTY, breaking arrow-key menus (prompts silently hang or auto-default).

- Navigate the interactive menu with arrow keys ↑↓ + Enter:

  - Action: Install

  - Product: Jetson

  - System configuration: Target Hardware

  - Target hardware: Jetson Orin Nano modules

  - SDK version: JetPack 7.2 (show all versions: Yes)

  - Install method: ISO Flash (Monitor) if a monitor is attached, else ISO Flash (Headless)

  - Additional SDKs: select Holoscan etc. if needed

It prints a final run command (execute as-is):

sdkmanager --action install --login-type devzone --product Jetson \
  --target-os Linux --version 7.2 --show-all-versions \
  --target JETSON_ORIN_NANO_TARGETS --install-method iso_flash \
  --additional-sdk 'Holoscan 4.3'

- Note: when the CLI’s --action install reaches the actual flash step, it relaunches the GUI itself, and at that point the GUI works fine. (Only direct GUI launch crashed.)

Fig. SDK Manager STEP 01 — JetPack 7.2 / Target Hardware selected

### 1-4. ISO flashing

- A step appears to burn the ISO onto an empty USB stick (8GB+) via balenaEtcher → plug in USB and proceed

- Plug that USB into the Jetson and boot → in the GRUB menu select “Install Jetson ISO 39.2.0” → auto-install

- ⚠️ Do not remove the USB before the completion signal — removing early hangs at EFI stub: Exiting boot services... (black screen). A power cycle recovers it, but it’s risky.

Fig. Burning the install USB with balenaEtcher 📷 [Photo: GRUB install menu “Install Jetson ISO 39.2.0” on the Jetson monitor]

Fig. SDK Manager STEP 04 — INSTALLATION COMPLETED SUCCESSFULLY

### 1-5. Create account + install SDK components

- After install the Jetson reboots → Ubuntu first-boot wizard → create an account (e.g. hs / 0000)

  - Skip the “check for updates” prompt (saves time)

- In the SDK Manager dialog on the host (“SDK components install”) enter that account + USB connection (IP 192.168.55.1) → CUDA/Holoscan etc. install automatically

- Done when it shows “INSTALLATION COMPLETED SUCCESSFULLY”

## 2. SSH into the Jetson from the host PC (work convenience)

You can work over the USB network without pulling the SD card each time.

# check connectivity
ping -c 2 192.168.55.1

# register SSH key (password-less login, one time)
ssh-copy-id hs@192.168.55.1   # enter password 0000

# connect
ssh hs@192.168.55.1

- Afterward you can run remote commands from the host: ssh hs@192.168.55.1 "command"

- For admin commands, pipe the password: echo 0000 | sudo -S command

## 3. Logo Replacement ① — UEFI firmware (most complex, source build required)

The logo shown right after power-on. It’s baked into the QSPI firmware as a BMP, so you must build UEFI from source.

Fig. (Before) UEFI stage — NVIDIA logo + firmware version

### 3-1. Docker build environment

sudo apt install docker.io
sudo usermod -a -G docker $USER   # add current user to the docker group
# → applies after logout/reboot (otherwise run with sudo docker)

### 3-2. Clone the UEFI source workspace

sudo mkdir -p /build && sudo chown $USER:$USER /build

# git identity is required (edkrepo clone fails without it)
git config --global user.name "yourname"
git config --global user.email "you@example.com"

IMG=ghcr.io/tianocore/containers/ubuntu-22-dev:latest

# init edkrepo config
sudo docker run --rm -v "$HOME":"$HOME" -e EDK2_DOCKER_USER_HOME="$HOME" $IMG init_edkrepo_conf
sudo docker run --rm -v "$HOME":"$HOME" -e EDK2_DOCKER_USER_HOME="$HOME" $IMG \
  edkrepo manifest-repos add nvidia https://github.com/NVIDIA/edk2-edkrepo-manifest.git main nvidia

# clone workspace (~400k objects; retry on an empty dir if the internet drops)
cd /build
sudo docker run --rm -w /build -v /build:/build -v "$HOME":"$HOME" \
  -e EDK2_DOCKER_USER_HOME="$HOME" $IMG \
  edkrepo clone nvidia-uefi NVIDIA-Platforms main

### 3-3. ⚠️ Replace the actual logo file (key pitfall)

It is LogoSingleBlack, not nvidiagray*.bmp (LogoMultipleGray), that is actually used. - Confirm in /build/nvidia-uefi/nvidia-config/t23x_general/.config: CONFIG_LOGO_IMPLEMENTER="Silicon/NVIDIA/Drivers/Logo/LogoSingleBlack.inf" - File to replace: Silicon/NVIDIA/Drivers/Logo/nvidiablack-1036x864.bmp (1036×864, black background)

Create the company logo at the same spec (1036×864, black bg, 24-bit BMP) and overwrite:

# Python (PIL) example — company logo PNG → 1036x864 black-bg BMP
from PIL import Image
src = Image.open('company_logo.png').convert('RGBA')
w, h = 1036, 864
canvas = Image.new('RGB', (w, h), (0,0,0))
tw = int(w*0.5); s = tw/src.width; th = int(src.height*s)
logo = src.resize((tw, th), Image.LANCZOS)
canvas.paste(logo, ((w-tw)//2, (h-th)//2), logo)
canvas.save('/build/nvidia-uefi/edk2-nvidia/Silicon/NVIDIA/Drivers/Logo/nvidiablack-1036x864.bmp', 'BMP')

### 3-4. Build UEFI

cd /build/nvidia-uefi
sudo docker run --rm -w /build/nvidia-uefi -v /build:/build -v "$HOME":"$HOME" \
  -e EDK2_DOCKER_USER_HOME="$HOME" $IMG \
  edk2-nvidia/Platform/NVIDIA/Tegra/build.sh \
  --init-defconfig edk2-nvidia/Platform/NVIDIA/Tegra/DefConfigs/t23x_general.defconfig

- Output: /build/nvidia-uefi/images/uefi_t23x_general_DEBUG.bin (~3MB; must be < 3.5MB UEFI partition)

### 3-5. Flash to QSPI (BSP required)

- developer.nvidia.com → Jetson Linux 39.2 → Driver Package (BSP) download (Jetson_Linux_R39.2.0_aarch64.tbz2, NOT Sources!)

- Extract and swap in your built binary:

- tar xjf ~/Downloads/Jetson_Linux_R39.2.0_aarch64.tbz2 -C /tmp/
cp /build/nvidia-uefi/images/uefi_t23x_general_DEBUG.bin \
   /tmp/Linux_for_Tegra/bootloader/uefi_bins/uefi_t23x_general.bin
sudo apt install -y libxml2-utils   # flash.sh needs xmllint

- Put the Jetson into recovery mode (see 1-2)

- ⚠️ Flash both A and B slots (flashing only A leaves it booting from B with no effect):

- cd /tmp/Linux_for_Tegra
# slot A
sudo ./flash.sh -k A_cpu-bootloader -c bootloader/generic/cfg/flash_t234_qspi.xml \
  jetson-orin-nano-devkit-super mmcblk0p1
# → re-enter recovery mode, then
# slot B
sudo ./flash.sh -k B_cpu-bootloader -c bootloader/generic/cfg/flash_t234_qspi.xml \
  jetson-orin-nano-devkit-super mmcblk0p1

  - This writes only a partition → the SD card’s OS is not wiped

📷 [Photo: flash.sh output “The [A_cpu-bootloader] has been updated successfully”]

Fig. (After) UEFI stage — company logo

## 4. Logo Replacement ② — Plymouth boot splash

The logo during kernel boot. (All of the below runs on the Jetson directly or via SSH.)

### 4-1. Create the custom theme

sudo mkdir -p /usr/share/plymouth/themes/company-logo

Prepare logo.png — black background, ~480px wide (the real Plymouth surface is ~1200px, so 480px ≈ 40% of screen):

from PIL import Image
src = Image.open('company_logo.png').convert('RGBA')
tw = 480; s = tw/src.width; th = int(src.height*s)
logo = src.resize((tw, th), Image.LANCZOS)
canvas = Image.new('RGB', (tw, th), (0,0,0))
canvas.paste(logo, (0,0), logo)
canvas.save('logo.png')

💡 Plymouth’s built-in scaler is low quality → pre-downscale to the exact size with PIL (LANCZOS) and place natively (no scaling) in the script for the sharpest result. Note: since the Plymouth framebuffer is ~1200px and the monitor upscales ~2x, it can’t physically be as sharp as the login logo (rendered at native 2560).

company-logo.plymouth:

[Plymouth Theme]
Name=Company Logo
ModuleName=script

[script]
ImageDir=/usr/share/plymouth/themes/company-logo
ScriptFile=/usr/share/plymouth/themes/company-logo/company-logo.script

company-logo.script (center, no scaling):

Window.SetBackgroundTopColor(0, 0, 0);
Window.SetBackgroundBottomColor(0, 0, 0);
logo.image = Image("logo.png");
logo.sprite = Sprite(logo.image);
logo.sprite.SetX(Window.GetWidth() / 2 - logo.image.GetWidth() / 2);
logo.sprite.SetY(Window.GetHeight() / 2 - logo.image.GetHeight() / 2);
fun refresh_callback() { }
Plymouth.SetRefreshFunction(refresh_callback);

### 4-2. Activate the theme

sudo ln -sf /usr/share/plymouth/themes/company-logo/company-logo.plymouth \
  /etc/alternatives/default.plymouth
echo -e "[Daemon]\nTheme=company-logo" | sudo tee /etc/plymouth/plymouthd.conf

### 4-3. ⚠️ Boot option + initrd pitfalls (the crux)

With default settings Plymouth won’t show on this image. Three things must be fixed:

(a) Kernel boot options — add splash quiet console=tty0 to the APPEND line in /boot/extlinux/extlinux.conf:

sudo cp /boot/extlinux/extlinux.conf /boot/extlinux/extlinux.conf.bak
sudo sed -i 's/console=ttyTCU0,115200/console=ttyTCU0,115200 console=tty0/' /boot/extlinux/extlinux.conf
sudo sed -i 's/video=efifb:off/video=efifb:off splash quiet/' /boot/extlinux/extlinux.conf

Without console=tty0, Plymouth thinks there’s “no display” and falls back to an empty text theme (required!)

(b) --graphical-boot option — add it to the initramfs script and the systemd units:

# initramfs script
sudo sed -i 's#--pid-file=/run/plymouth/pid#--pid-file=/run/plymouth/pid --graphical-boot#' \
  /usr/share/initramfs-tools/scripts/init-premount/plymouth

# systemd drop-ins (boot / reboot / shutdown screens)
for svc in plymouth-start plymouth-reboot plymouth-poweroff; do
  sudo mkdir -p /etc/systemd/system/$svc.service.d
  mode=boot; [ "$svc" = plymouth-reboot ] && mode=reboot; [ "$svc" = plymouth-poweroff ] && mode=shutdown
  printf '[Service]\nExecStart=\nExecStart=/usr/sbin/plymouthd --mode=%s --attach-to-session --graphical-boot\n' "$mode" \
    | sudo tee /etc/systemd/system/$svc.service.d/override.conf
done
sudo systemctl daemon-reload

Without --graphical-boot, plymouthd refuses the graphical splash because the console isn’t a VT

(c) Regenerate initrd — ⚠️ update-initramfs -u fails with “Nothing to do”. You must use mkinitramfs -o directly:

sudo mkinitramfs -o /boot/initrd $(uname -r)

Reboot and the Plymouth company logo appears.

Fig. Plymouth boot splash — company logo

## 5. Logo Replacement ③ — the post-login ~8s logo (.xsessionrc)

A full-screen NVIDIA logo for ~8s right after login, before the desktop appears. The cause is ~/.xsessionrc:

# this line paints the NVIDIA logo as the background via feh
feh --no-fehbg --bg-scale /usr/share/backgrounds/NVIDIA_Login_Logo.png

The white-background logo = this file. (The decisive clue distinguishing it from the black-background UEFI logo.)

Fix: replace that PNG with the company logo (1920×1080; black bg recommended for consistency):

from PIL import Image
src = Image.open('company_logo.png').convert('RGBA')
w, h = 1920, 1080
canvas = Image.new('RGB', (w, h), (0,0,0))   # black background
tw = int(w*0.4); s = tw/src.width; th = int(src.height*s)
logo = src.resize((tw, th), Image.LANCZOS)
canvas.paste(logo, ((w-tw)//2, (h-th)//2), logo)
canvas.save('NVIDIA_Login_Logo.png')

sudo cp /usr/share/backgrounds/NVIDIA_Login_Logo.png /usr/share/backgrounds/NVIDIA_Login_Logo.png.bak
sudo cp NVIDIA_Login_Logo.png /usr/share/backgrounds/NVIDIA_Login_Logo.png

⚠️ A sub-0.5s flicker may appear at the simpledrm→nvidia-drm handoff; this is closed NVIDIA driver behavior and is not fixable. Ignore it.

Fig. Post-login transition — company logo

Fig. Final desktop — GUI app (HVAC panel) auto-launched

## 6. Finishing touches (desktop wallpaper / auto-login / GUI app autostart)

### 6-1. Desktop + lock screen wallpaper

sudo cp company_bg.jpg /usr/share/backgrounds/company-bg.jpg
gsettings set org.gnome.desktop.background picture-uri 'file:///usr/share/backgrounds/company-bg.jpg'
gsettings set org.gnome.desktop.background picture-uri-dark 'file:///usr/share/backgrounds/company-bg.jpg'
gsettings set org.gnome.desktop.background picture-options 'zoom'
# create a marker so NVIDIA's default-background script won't overwrite it
mkdir -p ~/.local/share/applications && touch ~/.local/share/applications/nvbackground_ubuntu

### 6-2. Auto-login — /etc/gdm3/custom.conf

sudo sed -i 's/#  AutomaticLoginEnable = true/AutomaticLoginEnable = true/' /etc/gdm3/custom.conf
sudo sed -i 's/#  AutomaticLogin = user1/AutomaticLogin = hs/' /etc/gdm3/custom.conf

### 6-3. GUI app autostart — ~/.config/autostart/

mkdir -p ~/.config/autostart
cat > ~/.config/autostart/touch_panel.desktop <<'EOF'
[Desktop Entry]
Type=Application
Name=Jetson Touch Panel
Exec=/home/hs/jetson_touch_c/touch_panel
Path=/home/hs/jetson_touch_c
X-GNOME-Autostart-enabled=true
EOF

💡 An app using GPIO/SPI runs without sudo if the account is in the gpio group (check with groups). If a leftover root-launched process is holding GPIO lines, you get [GPIO] line N output request failed → clean it with sudo pkill -9 appname.

## 7. Cloning the SD card (for mass-producing identical boards)

You can copy this configured SD card to other identical boards. - Image the whole card with dd or balenaEtcher and write it to a new SD card (same or larger capacity) → behaves identically - ⚠️ If multiple boards are on the same network simultaneously, identical hostname/SSH keys collide → change with sudo hostnamectl set-hostname newname on each board - The UEFI (QSPI) firmware lives on the board, not the SD card, so each new board must be UEFI-flashed (section 3) separately for the UEFI logo to change. (SD card cloning only carries the Plymouth/login/desktop logos.)

## 8. Pitfalls Summary (the time sinks)

| Pitfall | Fix |
| --- | --- |
| SDK Manager GUI crashes immediately | Work around with CLI mode (--query interactive), no pipes |
| UEFI logo won’t change | It’s not LogoMultipleGray — the real one is LogoSingleBlack/nvidiablack-1036x864.bmp |
| Flashed but logo unchanged | Must flash both A and B slots |
| Plymouth won’t show | Needs both console=tty0 and --graphical-boot |
| initrd not updating | Use mkinitramfs -o, not update-initramfs -u |
| NVIDIA logo after login | Not GDM/extensions — it’s feh in ~/.xsessionrc |
| Plymouth logo blurry/wrong size | Pre-downscale to ~480px with PIL + native placement (avoid Plymouth’s scaler) |

Authored: 2026-06-24 / JetPack 7.2 (L4T R39.2.0) / Jetson Orin Nano Super Devkit